UNICA TOP. The top range fully automatic diemaking machine Unica Top is positioned at the top of the "all in one" machines for the diemakers. In this one machine you will find all the functions for the diemaker: Straight cut/Lip Bridges Broaching Bending Perforating rule Cut/crease rule Nick grinding The Unica Top has been designed using all of our expertise gained from over 10 years of manufacturing automatic diemaking machines. The machine is a natural evolution from our current range with brand new features as well as improved features. Unica Top has been especially designed to meet the most demanding bending requirements of the diemaking industry, mainly for the diemakers producing high quality dies for the folding carton and corrugated box manufacturers. Unica Top can process a complete range of steel rule from 18,00 to 30,00 mm high and 1.5, 2 and 3 Pt thick (0.5, 0.71 and 1.05 mm). Designed to work in the dieshops dedicated to the manufacturing of high quality dies UnicA Top is fully integrated with all the other machines of the SDDS (Serviform Digital Diemaking System). All the machines of the SDDS (Serviform Digital Diemaking System) are driven by a tailored control software based Upon Windows XP especially developed for the diemaking industry. Main features of the software are: - Ease of use - Perfect interface ability with existing networks - Extreme flexibility Continuing with the Serviform tradition of fully interchangeable bending heads, the Unica Top allows you to fully work with different thickness and rule heights. With the changeover from one rule thickness/height to another completed within minutes, it makes this Unica Top the most versatile and productive auto bender available. All the punching operations are performed by three multifunction cartridges. Pneumatically driven, these cartridges allow for multiple operations (i.e. cutting, lipping, notching, bend marking, deformation) saving on precious space and increasing production speed. Basic Unica Top cartridge configuration includes special cartridges for producing perforation and cut/crease rule out of standard cutting rule. The cutting cartridge and the cartridge for "special" operation are motorised and allow pre selection using the control software the height of the creasing rule to be used or the selection with high degree of accuracy the height of the creasing sections of the cut/crease rule to be manufactured. Unica Top is supplied with a completely re-designed dual operation in-line broaching unit. Now broaching is faster, cleaner and burr free. Thanks to the broaching of rule, minimum radiuses are now smaller and the quality level of the dies obtained are increased. A revolutionary grinding unit for nicks produces perfect geometric & uniform nicks all in a straight line process. Three electric spindles equipped with diamomd grinding wheels are driven by the machine control software: the diemaker is sure to obtain burr free nicks absolutely accurate and with constant depth. The position of the nicks is automatically importaed from the CAD drawing. Manufacturing time for the dies becomes shorter and shorter! TECHNICAL SPECIFICATIONS Hardware IBM Compatible Data input CFF 2, DDES, .N, DXF. Tailored interface available on request Steel rule thickness 1.5 Pt. (0.5 mm), 2 Pt. (0.71 mm), 3 Pt. (1.05 mm) Steel rule height 18,00 ~ 30.00 mm Power supply 400V 3 Ph., 4 kW or 220V 3 Ph., 4 kW (different supplie available on request) Air pressure 6 bar Dimensions Machine body: 1800 x 700 x 1600 mm; Coil holder trolley: 700 x 700 x 1000 mm Weight Machine body: 730 kg Coil holder trolley: 70 kg Technical specifications can change without notice....